1)DC MOTOR

- Commonly used for a mobile

robot base like differential drive,

omni-wheeled and

mecanum wheeled robot.

- It also usually being used for any

rotational movement that require

high speed or high torque.

ADVANTAGES:

- Precise Speed Control: Easily adjustable speed for various applications.

- High Starting Torque: Ideal for tasks requiring strong initial force.

- Simplicity and Cost: Simple design and control, leading to lower initial costs and easier maintenance.

- Versatility: Suitable for a wide range of applications.

- Quick Response: Rapid response to control signals.

- High Efficiency: Particularly efficient at lower speeds.

DISADVANTAGES:

- Maintenance Needs: Regular upkeep for brushes and commutator.

- Size and Weight: Larger and heavier compared to AC motors of the same power.

- Component Costs: Brushes and commutator add to overall cost.

- Voltage Limitations: Generally suited for low voltage applications.

- Reduced Lifespan: Shorter lifespan due to mechanical wear.

- Sparking Issues: Not suitable for hazardous environments due to potential sparking.

WIRING DIAGRAM :

SAMPLE CODE :

// Define the pins

const int dirPin = 8;

const int pwmPin = 9;

void setup() {

// Set the direction pin as an output

pinMode(dirPin, OUTPUT);

// Set the PWM pin as an output

pinMode(pwmPin, OUTPUT);

}

void loop() {

// Turn the motor clockwise

digitalWrite(dirPin, HIGH);  // Set direction to clockwise

analogWrite(pwmPin, 255);    // Set speed (0-255)

// Wait for 2 seconds

delay(2000);

// Stop the motor

analogWrite(pwmPin, 0);      // Set speed to 0

// Wait for 1 second

delay(1000);

// Turn the motor counterclockwise

digitalWrite(dirPin, LOW);   // Set direction to counterclockwise

analogWrite(pwmPin, 255);    // Set speed (0-255)

// Wait for 2 seconds

delay(2000);

// Stop the motor

analogWrite(pwmPin, 0);      // Set speed to 0

// Wait for 1 second

delay(1000);

}

2) SERVO MOTOR

- Commonly used to control the joints

of robotic arms, allowing for precise

positioning and movement of the

arm segments.

- It also being used to control the

opening and closing of grippers

or other end effectors on the robot,

enabling it to grasp and manipulate objects.

ADVANTAGES:

- Precision and Accuracy: Servo motors provide precise control of angular or linear position, velocity, and acceleration, making them ideal for applications requiring high accuracy.

- High Torque: They can generate high torque at low speeds, which is beneficial for applications needing strong and controlled movement.

- Speed Control: Servo motors offer excellent speed control, enabling smooth and precise motion profiles.

- Closed-Loop System: The built-in feedback system (usually involving encoders) allows for accurate positioning and speed control, as well as error correction.

- Efficiency: They are highly efficient in converting electrical energy into mechanical energy, leading to less heat generation and lower energy consumption.

- Versatility: Available in various sizes and configurations, making them suitable for a wide range of applications from small hobby projects to industrial machinery.

DISADVANTAGES:

- Cost: Servo motors are generally more expensive compared to other types of motors, such as stepper motors.

- Complexity: The control systems for servo motors are more complex, requiring specialized knowledge and equipment to implement and maintain.

- Maintenance: They often require more maintenance due to the presence of brushes (in brushed servo motors) and the complexity of the feedback system.

- Limited Rotation: Most servo motors are designed for limited rotation (typically 180 degrees or less), which may not be suitable for applications requiring continuous rotation.

- Heat Dissipation: At high loads, servo motors can generate significant heat, necessitating cooling systems to prevent overheating.

WIRING DIAGRAM:

SAMPLE CODE :

#include <Servo.h>

Servo myservo;  // create servo object to control a servo

// twelve servo objects can be created on most boards

int pos = 0;    // variable to store the servo position

void setup() {

myservo.attach(9);  // attaches the servo on pin 9 to the servo object

}

void loop() {

for (pos = 0; pos <= 180; pos += 1) { // goes from 0 degrees to 180 degrees

// in steps of 1 degree

myservo.write(pos);              // tell servo to go to position in variable 'pos'

delay(15);                       // waits 15ms for the servo to reach the position

}

for (pos = 180; pos >= 0; pos -= 1) { // goes from 180 degrees to 0 degrees

myservo.write(pos);              // tell servo to go to position in variable 'pos'

delay(15);                       // waits 15ms for the servo to reach the position

}

}

3) DYNAMIXEL SERVO

- Robotic Arms: Dynamixel servos are frequently used in the joints of robotic arms to provide smooth and precise movements.

- Humanoid Robots: In humanoid robots, these servos are used in the joints of limbs, including elbows, knees, shoulders, and hips, to mimic human motion.

- Quadrupeds and Hexapods: Dynamixel servos are used in the legs of these robots to provide locomotion capabilities.

- Claws and Grippers: For industrial robots, Dynamixel servos are used in the actuation of claws and grippers to handle objects.

ADVANTAGES:

- High Precision and Control : Real-time feedback on position, speed, temperature, and load. High-resolution control for accurate and smooth movements.

- Ease of Integration : Daisy-chain capability simplifies wiring and control architecture. Compatibility with various communication protocols (e.g., TTL, RS485).

- Programmability : Customizable parameters such as speed, torque, and position limits.

- Durability and Reliability : Robust design ensures longevity and reliability in various conditions.

- Versatility : Wide range of models suitable for different applications, from small hobby projects to large industrial robots.

DISADVANTAGES:

- Cost : Generally more expensive than standard servos, which can be a limiting factor for budget-conscious projects.

- Complexity : Higher learning curve requiring advanced programming skills, challenging for beginners.

- Power Requirements : Higher power consumption compared to standard servos, requiring robust power management.

- Maintenance : Potential for overheating under heavy loads, necessitating proper cooling and maintenance.

WIRING DIAGRAM:

SAMPLE CODE:

#include <SoftHalfDuplexSerial.h>

#include <DynamixelAx.h>

softHalfDuplexSerial port(8);  // data pin 8

dxlAx dxlCom(&port);

String _readString;  // Input string from serial monitor

bool _strComplete = false;

int _id = 1;  // Default Dynamixel servo ID

void printServoId(String msg);

void printDxlResult();

void printDxlError(unsigned short dxlError);

void setup() {

// Open serial communications and wait for port to open (PC communication)

Serial.begin(57600);

while (!Serial) {

;  // wait for serial port to connect. Needed for native USB port only

}

Serial.println("Starting COM!");

dxlCom.begin(57600);

}

/////////////////////////////////////////////////////////////////////////////////////

void loop() {

while (Serial.available()) {

char inputChar = Serial.read();  //gets one byte from serial buffer

_readString += inputChar;        //makes the string readString

if (inputChar == '\n')

_strComplete = true;

}

if (_strComplete) {

_strComplete = false;

if (_readString.startsWith("ID")) {

_readString.remove(0, 2);

_id = _readString.toInt();  //convert readString into a number

printServoId("Communicating with");

Serial.println(_id);

} else if (_readString.startsWith("ping")) {

printServoId("Ping");

dxlCom.ping(_id);

printDxlResult();

} else if (_readString.startsWith("action")) {

dxlCom.isRegistered(_id);

while (!dxlCom.dxlDataReady())

;  // waiting the answer of servo

printDxlError(dxlCom.readDxlError());

if (dxlCom.readDxlResult())  // if it is registred

{

printServoId("Execute reg command in");

while (dxlCom.isBusy())

;  // waiting the status return delay time

dxlCom.action(_id);

printDxlResult();

} else {

printServoId("No reg command in");

Serial.println();

}

} else if (_readString.startsWith("reboot")) {

printServoId("Reboot (not supported by MX)");

dxlCom.reboot(_id);

printDxlResult();

} else if (_readString.startsWith("model")) {

printServoId("Model number of");

dxlCom.readModelNumber(_id);

printDxlResult();

} else if (_readString.startsWith("firmware")) {

printServoId("Firmware number of");

dxlCom.readFirmware(_id);

printDxlResult();

} else if (_readString.startsWith("setID")) {

_readString.remove(0, 5);

int newID = _readString.toInt();  //convert readString into a number

printServoId("Setting");

Serial.print(_id);

Serial.print(" to ");

Serial.print(newID);

Serial.print(" : ");

dxlCom.setId(_id, newID);

printDxlResult();

} else if (_readString.startsWith("led")) {

_readString.remove(0, 3);

bool Status = _readString.toInt();  //convert readString into a number

printServoId("Changing led status of ");

dxlCom.setLedEnable(_id, Status);

printDxlResult();

} else if (_readString.startsWith("move")) {

_readString.remove(0, 4);

unsigned short Position = _readString.toInt();  //convert readString into a number

printServoId("Moving ");

dxlCom.setGoalPosition(_id, Position);

printDxlResult();

bool isMoving = true;

while (isMoving) {

unsigned short error = DXL_ERR_SUCCESS;

while (dxlCom.isBusy())

;  // waiting the status return delay time

dxlCom.readPresentPosition(_id);

Serial.print("Pos : ");

while (!dxlCom.dxlDataReady())

;  // waiting the answer of servo

error = dxlCom.readDxlError();

if (error != DXL_ERR_SUCCESS)  // readDxlResult should always be called before readDxlData

printDxlError(error);

Serial.println(dxlCom.readDxlResult());

while (dxlCom.isBusy())

;  // waiting the status return delay time (for testing if it is moving)

dxlCom.isMoving(_id);

while (!dxlCom.dxlDataReady())

;  // waiting the answer of servo

error = dxlCom.readDxlError();

if (error != DXL_ERR_SUCCESS)  // readDxlResult should always be called before readDxlData

printDxlError(error);

isMoving = dxlCom.readDxlResult();

}

} else if (_readString.startsWith("speed")) {

_readString.remove(0, 5);

unsigned short Speed = _readString.toInt();  //convert readString into a number

printServoId("Set speed of ");

dxlCom.setMovingSpeed(_id, Speed);

printDxlResult();

} else if (_readString.startsWith("torque")) {

_readString.remove(0, 6);

unsigned short torque = _readString.toInt();  //convert readString into a number

printServoId("Set torque of ");

dxlCom.setTorqueLimit(_id, torque);

printDxlResult();

} else if (_readString.startsWith("voltage")) {

printServoId("Voltage (to be divided by 10) of");

dxlCom.readVoltage(_id);

printDxlResult();

} else if (_readString.startsWith("temperature")) {

printServoId("Temperature of");

dxlCom.readTemperature(_id);

printDxlResult();

} else if (_readString.startsWith("regmove")) {

_readString.remove(0, 7);

unsigned short Position = _readString.toInt();  //convert readString into a number

printServoId("Write command (type 'action' to execute) in REG register of ");

dxlCom.sendDxlRegData(_id, DXL_ADD_GOAL_POSITION, (const byte*)&Position, 2);

printDxlResult();

}

_readString = "";  //empty for next input

}

}

/////////////////////////////////////////////////////////////////////////////////////

void printDxlResult() {

while (!dxlCom.dxlDataReady())

;  // waiting the answer of servo

printDxlError(dxlCom.readDxlError());

Serial.println(dxlCom.readDxlResult());

}

void printServoId(String msg) {

Serial.print(msg);

Serial.print(" servo ID ");

Serial.print(_id);

Serial.print(" - ");

}

void printDxlError(unsigned short dxlError) {

// after any operation error can be retrieve using dx::readDxlResult() (i.e. after read or write operation)

if (dxlError == DXL_ERR_SUCCESS)

Serial.println("OK");

else {

if (dxlError & DXL_ERR_VOLTAGE)

Serial.print("voltage out of range-");

if (dxlError & DXL_ERR_ANGLE)

Serial.print("angle out of range-");

if (dxlError & DXL_ERR_OVERHEATING)

Serial.print("overheating-");

if (dxlError & DXL_ERR_RANGE)

Serial.print("cmd out of range-");

if (dxlError & DXL_ERR_TX_CHECKSUM)

Serial.print("Tx CRC invalid-");

if (dxlError & DXL_ERR_OVERLOAD)

Serial.print("overload-");

if (dxlError & DXL_ERR_INSTRUCTION)

Serial.print("undefined instruction-");

if (dxlError & DXL_ERR_TX_FAIL)

Serial.print("Tx No header-");

if (dxlError & DXL_ERR_RX_FAIL)

Serial.print("Rx No header-");

if (dxlError & DXL_ERR_TX_ERROR)

Serial.print("Tx error-");

if (dxlError & DXL_ERR_RX_LENGTH)

Serial.print("Rx length invalid-");  // Not implemented yet

if (dxlError & DXL_ERR_RX_TIMEOUT)

Serial.print("timeout-");

if (dxlError & DXL_ERR_RX_CORRUPT)

Serial.print("Rx CRC invalid-");

if (dxlError & DXL_ERR_ID)

Serial.print("Wrong ID answered-");  // ?? Hardware issue

Serial.println();

}

}

4) STEPPER MOTOR

- Robotic Arms: Stepper motors are often used to control the movement of joints in robotic arms, providing precise positioning and repeatability necessary for tasks such as assembly, welding, or painting.

    - Humanoid Robots: In humanoid robots, stepper motors can be used in the joints to mimic human-like movements with a high degree of accuracy.

    - Linear Actuators: Frequently used in linear actuators for tasks that require precise linear movement, such as in pick-and-place operations or 3D printing.

    - XY Tables: In CNC machines and 3D printers, stepper motors control the movement of the print head or cutting tool along the X and Y axes.

ADVANTAGES:

- Precision and Accuracy : Provides precise control over movement without needing feedback systems. Consistently returns to the same position with high accuracy.

- Ease of Control : Can be controlled without complex feedback systems, simplifying the control mechanism. Operates effectively with simple digital control signals.

- Reliability : Typically, stepper motors are brushless, reducing mechanical wear and increasing longevity. Less prone to mechanical failure, resulting in lower maintenance needs.

- Holding Torque : Can hold its position firmly when not moving, making it useful in applications where maintaining a fixed position is critical.

- Cost-Effective : Generally less expensive than servomotors, especially for applications not requiring closed-loop control.

DISADVANTAGES:

- Efficiency and Heat : Can consume more power, especially at higher speeds, leading to inefficiency. Heat Generation: Tends to generate significant heat during operation, which can necessitate additional cooling mechanisms.

- Torque and Speed Limitations : Loses torque as the speed increases, which can limit its use in high-speed applications. Generally not suitable for applications requiring very high speeds.

- Resonance and Vibration : Can exhibit resonant frequencies that cause vibrations, potentially affecting performance and requiring dampening techniques. Often noisier than other motor types due to the stepping action.

- Control Complexity : Achieving smoother motion and higher resolution through microstepping can complicate control systems. May require sophisticated driving circuits for optimal performance.

WIRING DIAGRAM:

SAMPLE CODE:

//Includes the Arduino Stepper Library

#include <Stepper.h>

// Defines the number of steps per rotation

const int stepsPerRevolution = 2038;

// Creates an instance of stepper class

// Pins entered in sequence IN1-IN3-IN2-IN4 for proper step sequence

Stepper myStepper = Stepper(stepsPerRevolution, 8, 10, 9, 11);

void setup() {

// Nothing to do (Stepper Library sets pins as outputs)

}

void loop() {

// Rotate CW slowly at 5 RPM

myStepper.setSpeed(5);

myStepper.step(stepsPerRevolution);

delay(1000);

// Rotate CCW quickly at 10 RPM

myStepper.setSpeed(10);

myStepper.step(-stepsPerRevolution);

delay(1000);

}

5) BRUSHLESS MOTOR

- Actuators: Used in robotic joints, grippers, and manipulators.

- Wheels and Drivetrains: Drive the wheels or tracks of mobile robots.

- Propulsion Systems: Power propellers in aerial drones.

- Conveyor Systems: Operate conveyor belts in industrial robots.

- Servo Motors: Provide precise control in applications like CNC machines and robotic surgery.

- Gimbals and Stabilizers: Stabilize cameras in drones and handheld devices.

- Linear Actuators: Convert rotational motion into linear motion for precise linear movement.

ADVANTAGES:

- Higher Efficiency: Brushless motors have better efficiency compared to brushed motors, resulting in less energy loss and heat generation.

- Longer Lifespan: They have fewer mechanical parts (no brushes), which reduces wear and tear and increases durability.

- Lower Maintenance: The absence of brushes means less frequent maintenance and replacements are required.

- Higher Performance: Brushless motors provide higher torque-to-weight ratio and can achieve higher speeds.

- Better Reliability: They offer more consistent performance and reliability under various operating conditions.

- Reduced Noise: Brushless motors operate more quietly due to the absence of brushes.

- Precise Control: They offer more precise control of speed and position, which is critical for applications requiring high accuracy.

DISADVANTAGES:

- Higher Cost: Brushless motors are generally more expensive to produce and purchase compared to brushed motors.

- Complexity: The control systems for brushless motors are more complex, requiring sophisticated electronics and software.

- Initial Setup: They often need more intricate initial setup and tuning to achieve optimal performance.

- Electronic Speed Controllers (ESCs): Brushless motors require ESCs, which add to the cost and complexity of the system.

- Size and Weight: In some applications, the additional components for controlling brushless motors might add to the size and weight of the system.

WIRING DIAGRAM:

SAMPLE CODE:

#include <Servo.h>

Servo ESC;     // create servo object to control the ESC

int potValue;  // value from the analog pin

void setup() {

// Attach the ESC on pin 9

ESC.attach(9,1000,2000); // (pin, min pulse width, max pulse width in microseconds)

}

void loop() {

potValue = analogRead(A0);   // reads the value of the potentiometer (value between 0 and 1023)

potValue = map(potValue, 0, 1023, 0, 180);   // scale it to use it with the servo library (value between 0 and 180)

ESC.write(potValue);    // Send the signal to the ESC

}

6) LINEAR ACTUATOR

- Grippers and End Effectors: Linear actuators are often used to control the opening and closing of robotic grippers or other end effectors that interact with objects.

- Lifting Mechanisms: Robots designed to lift objects, such as warehouse robots or robotic arms, use linear actuators to raise and lower payloads.

- Conveyance Systems: Linear actuators are used in robots that move objects along a conveyor belt or similar systems, ensuring precise placement and movement.

- Adjustable Bases: For robots that need adjustable bases or platforms, linear actuators provide the necessary vertical or horizontal adjustments.

ADVANTAGES:

- Precision and Control : Linear actuators provide precise control over linear movement, making them ideal for applications requiring accurate positioning. They offer consistent performance with high repeatability, which is crucial for repetitive tasks.

- Force and Load Capacity : Capable of exerting significant force, making them suitable for lifting and moving heavy loads. Available in various sizes and capacities to match specific application requirements.

- Versatility : Used in diverse applications from industrial machinery to consumer electronics. Can be integrated into various systems and with different types of controllers.

- Efficiency and Maintenance : Many linear actuators, especially electric ones, are energy efficient. Generally, they have fewer moving parts compared to other systems, resulting in lower maintenance needs.

- Simple Design and Installation : Their compact design allows for easy integration into tight spaces. Relatively straightforward to install and set up.

DISADVANTAGES:

- Speed Limitations : Typically, linear actuators are slower compared to rotary actuators or other motion systems.

- Cost : High-quality linear actuators can be expensive. Custom or specialized actuators for specific tasks can significantly increase costs.

- Complexity and Size : May require sophisticated control systems and electronics for precise operation. Some actuators can be bulky, limiting their use in compact applications.

- Environmental Sensitivity : Performance can be affected by extreme temperatures or harsh environmental conditions, requiring additional protection or specific models designed for such environments.

- Limited Range of Motion : Linear actuators are limited to a specific range of motion (stroke length), which might not be suitable for all applications.

WIRING DIAGRAM:

SAMPLE CODE:

byte Speed = 0; // Intialize Varaible for the speed of the motor (0-255);

int RPWM = 10;  //connect Arduino pin 10 to IBT-2 pin RPWM

int LPWM = 11;  //connect Arduino pin 11 to IBT-2 pin LPWM

void setup() {

pinMode(10, OUTPUT); // Configure pin 10 as an Output

pinMode(11, OUTPUT); // Configure pin 11 as an Output

}

void loop() {

// Extend Actuator at Full Speed

Speed = 255;

analogWrite(RPWM, 0);

analogWrite(LPWM, Speed);

delay(2000); // 2 Seconds

// Stop Actuator

analogWrite(RPWM, 0);

analogWrite(LPWM, 0);

delay(2000); // 2 Seconds

// Retract Actuator at Half Speed

Speed = 127;

analogWrite(RPWM, Speed);

analogWrite(LPWM, 0);

delay(2000); // 2 Seconds

// Stop Actuator

analogWrite(RPWM, 0);

analogWrite(LPWM, 0);

delay(2000); // 2 Seconds